MVR Distillation

Model & Control Reference

MCC DAQ expression-engine control + plant simulator

Generated from server/config/expressions_MVR.json

# Contents

# 1.  Overview

This is the MVR (mechanical vapor recompression) water-distillation control. The control laws run inside the MCC DAQ app's expression engine every tick (~100-160 Hz). A built-in plant simulator (MVRSim) lets you develop and tune the controls without the hardware: set simEnable=1 to feed the controllers from the model instead of the real AI/TC.

Two backends evaluate the expressions identically: the Python engine and the compiled C++ DLL. Top-level static.x = const lines initialize once.

Tick data flow: MVR System (phase) -> SensorMux (real OR sim -> y*) -> SteamTables -> Interlocks -> heater PIs -> BlowerControl / FeedwaterControl -> CondensatePump -> ProductionControl / FeedFollow -> MVRSim (plant, sim only).

Sensor abstraction: every sensor is a static.yXxx. SensorMux sets them from the real AI/TC when simEnable=0, or from the sim model when =1, so the control laws are identical in both modes.

# 2.  Control Laws

These run regardless of sim vs real. Heater outputs are PWM duty 0..1; temps degC; pressures psia.

TIMING: the server stamps the system monotonic clock (ms) into static.sysMs every tick before the expressions evaluate. Setpoints differences it into static.dtReal = (sysMs - lastTickMs)/1000 -- the REAL elapsed time, which every timer/integral/window uses instead of the nominal tick period. In sim it is accelerated as dtReal * simSpeed; in measure mode it is the true elapsed seconds. timeIncSec is only the fallback (first ticks / hiccups).

## 2.1  Startup state machine (MVR System)

IDLE -> PURGE -> PRIME -> HEAT -> RUN. heatOK and the blower are gated on phase>=HEAT so the makeup element never dry-fires.

- PURGE (full / dry start): reverse the feed pump (vel = -purgeRevRpm, 400 rpm) until the CALIBRATED sender reads BOTTOM (plateau 0), then keep pumping purgeExtraML (200 ml) more -- water below the float pickup is still in there. If the sender is faulted, fall back to the blind purgeMaxML dead-reckon cap. A WATCHDOG (purgeStallS) trips purgeErr if the level stops dropping (pump/direction fault); it disarms once bottom is reached (the overrun is legitimately flat).

- PURGE (WARM START, purgeWarmEn=1): if the sender already shows water at Run, do NOT purge to dry: pump down until the level has dropped at least ONE plateau AND reads at/below the operating mark. Every plateau-drop crossing snaps feedInvEst to the edge midpoint (the fusion), so startup continues from a CALIBRATED level -- a restart from an overfull shutdown pumps down to the mark, and coming from above the LAST crossing is the 2600|2350 edge = exactly the 2475 operating level. Minutes instead of a full drain + refill.

- PRIME: feedCmd = feedPrimeRpm (250 rpm); fill to the EvapLow mark (~0.75 L, heater covered) and advance to HEAT when feedInvEst >= feedPrimeML AND yEvapLow -- the low switch CONFIRMS wet, so the heater never fires dry. A warm start that lands above the low mark falls straight through. If primed but EvapLow stays dry by evapLevelCheckML, Interlocks latches evapLevelErr (feed-pump or sensor failure) -> trip + UI popup; clears on Stop.

- HEAT: makeup heater on; the feed RAMPS the level up to the operating mark (2475). The blower stays OFF until yEvapTemp >= blowerStartTemp (90 C), then soft-starts (2.10). Advance to RUN when yEvapTemp > evapTempSet - 3 AND yEvapMid -- near boiling AND at the operating level (seed prodMeas = prodSetSafe so the blower doesn't spike).

- RUN: closed-loop production; the feed HOLDS the level at the 2475 edge (see 2.2a).

- EMPTY / SHUTDOWN (buttons, Run OFF): Button widgets in Variable mode named emptySystem or shutdown reverse-pump the system dry without a run: sender bottom + purgeExtraML, emptyMaxML (4000) backstop. With Run off the heaters are already forced off (heatOK=0 -> HeaterEnable + both PWMs = 0) and the blower is disabled, so shutdown = verified-heaters-off + pump-out. Ignored while running; toggle off to re-arm; hitting Run aborts it.

## 2.2a  Evaporator level control (FeedFollow + the sender fusion)

The tank-gauge sender IS the level sensing -- a 14-step STAIRCASE (plateau centers from the measured cal table), so the raw reading cannot resolve the level within a plateau (~200-350 ml wide). The control fuses it with the calibrated positive-displacement feed pump: feedInvEst dead-reckons feed-in minus vapor-out between plateaus; every plateau CHANGE is a known physical edge crossing that SNAPS the estimate to the old/new plateau midpoint (and teaches a slow rate-bias levelBiasLpm, clamp +/-0.05 L/min); between crossings the estimate is clamped inside plateau-center +/- senderSpanML (no teaching -- centers are only approximate).

Feed = measured-production feed-forward prodMeas x (1+blowdownFrac) + P on the fused estimate OUTSIDE a +/-levelDeadML deadband (fast fill / upset recovery) + a slow EvapMid-switch integral INSIDE the deadband (the tight hold: the 2475 edge is a perfect 1-bit sensor placed exactly at the setpoint). Integrating only where P is silent stops the two from fighting. A HEAT-phase guard keeps filling while the mid switch is dry regardless of the estimate, and yEvapHigh cuts the feed entirely. The marks sit ON plateau edges (750 / 2475 / 2750) so each switch flip is a real crossing. Calibrate condCalFactor once against the feed pump so production reads true.

## 2.2  Setpoint safety bounds

Each live setpoint is range-checked every tick; an out-of-range value is ignored and the last valid value is kept in a *Safe companion. The controllers read the *Safe value, never the raw input.

if (lo <= setpoint <= hi)  ->  setpointSafe = setpoint   ; else keep last

Guarded: productionSet [0,prodSetMax], superheatSet [0,superheatSetMax], evapTempSet [min,max], blowerRpmSet [0,blowerMaxRpm], feedRpmSet [0,feedRpmMax].

## 2.3  Interlocks & trips (Interlocks)

trip = any of: evap/steam over-pressure, vapor-out over-temp, demister flood (QUALIFIED: DemisterFlood AND DemisterWater both wet -- fog films the flood sensor alone), low-superheat (wet compression; arms only at target blower rpm), makeup-cartridge over-temp (HARD backstop makeupHtrMax=140C), superheat-cartridge over-temp, evaporator-level prime-verify (evapLevelErr), level-sender fault (senderErr: open/short mid-run, debounced), purge stall (purgeErr: watchdog). All the latched trips clear when runSystem cycles; evapLevelErr, senderErr, purgeErr, and the high-level warning also fire UI popups. The HIGH-LEVEL warning (evapHighWarn, 2750) is latched + feed-cut but NOT a trip, and does not latch during PURGE (an overfull warm start is legitimately high while draining).

Makeup-heater element protection is SOFT first: in MakeupControl, above makeupHtrSoft (115C) the PWM duty is folded back proportionally (violent boil / local dry-out -> ease off so water re-wets the element), never below the keep-warm floor makeupHtrMinDuty. Only if the element keeps climbing to makeupHtrMax (140C) despite fold-back does the hard trip fire. Tune makeupHtrMinDuty to the PWM that just holds the vapor at ~100C.

heatOK = runSystem AND (trip == 0) AND (mvrPhase >= HEAT)

makeupHtrTrip latches if  yMakeupHtrTemp > makeupHtrMax   (uncovered / feed overheat)

superHtrTrip  latches if  ySuperHtrTemp  > superHtrMax

## 2.4  Makeup heater PI (MakeupControl)

Holds the evaporator at the boiling setpoint. Conditional anti-windup; gated by heatOK.

err  = evapTempSetSafe - yEvapTemp

I    = clamp( I + kiMakeup * err * dt , 0 , 1 )

duty = clamp( kpMakeup * err + I , 0 , 1 )      ->  DO:MakeupHtr  (makeupDuty)

## 2.5  Superheat heater -- element-temp regulation + PI (SuperheatControl)

Rig finding: the cartridge couples heat into thin vapor POORLY -- dry/low-flow, the surface just soars (the old 180C trip fired before any superheat showed). The duty is now the MIN of two demands: (1) the superheat PI -- the real control, meaningful once vapor flows -- and (2) an element-temp P regulator holding the cartridge TC at superHtrRegC (200C). From the moment the blower is turning (blowerCmdRpm >= blowerRampRpm0) the element preheats REGULATED at ~200C; as vapor flow builds it carries the heat away, the element cools, the regulator releases, and the superheat PI takes over seamlessly. The regulation IS the dry-fire protection; superHtrMax (250C) is the hard backstop. The PI integral holds while the regulator limits (anti-windup); both heater integrators reset whenever their heater is off. The wet-compression (low-superheat) trip arms only once the blower reaches its target.

pi   = clamp( kpSuper*(superheatSetSafe - superheatIn) + I , 0 , 1 )

elem = clamp( kpShElem * (superHtrRegC - ySuperHtrTemp) , 0 , 1 )

duty = min( pi , elem )       ->  DO:SuperHtr  (superDuty)

## 2.6  Blower production control (ProductionControl)

Trims the blower (the compressor / vapor pump) to hold the target distillate rate. Measured production now comes from the condensate BATCHES (see 2.9): each pump-to-dry cycle meters a known distillate volume, and volume / fill-time is the average production rate -- the only rate a single point-level sensor can give. Updated once per empty cycle (condRateLpm) and held between; before the first batch the blower holds nominal rpm. cdt accelerates the integral in sim so it converges at the same wall-clock feel.

measProd = condRateLpm                         (batch volume / fill time)

prodMeas += (measProd - prodMeas) * smoothing

blowerRpmSet = clamp( blowerRpmSet + kProd*(prodSetSafe - prodMeas)*cdt , min , max )

Blower estimate (used while venting and for feedInvEst): blowerProdEst = prodPerRpm * blowerRpmSafe * clamp((eT-95)/5,0,1).

## 2.7  Feed level control (FeedFollow + FeedwaterControl)

The stepped tank-gauge sender fused with the calibrated feed pump (see 2.2a). Far below the operating mark P fills fast; inside the deadband the EvapMid edge integral holds the level. blowdownFrac = 0 (no blowdown valve), so the feed-forward is just the production rate:

feedLpm = prodMeas*(1+blowdownFrac) + P(fused est, outside deadband) + feedLevelTrim

feedLevelTrim += kFeedLevelI * (EvapMid dry ? +1 : -1) * dt      (INSIDE the deadband only)

feedRpmSet = clamp( feedLpm / (stepsPerRev*LPerStep) , 0 , feedRpmMax )   -> STEP:Feed.VELOCITY

Velocity is driven as a LEVEL (0 = stop) so every run/stop transition re-triggers the stepper move. The edge integral -- not the dead-reckon -- closes the tight loop, so a mis-calibrated flow meter can never run the level away ('wet' always pulls feed below boil-off).

COLD-CRASH GUARD (RUN only): a max-feed level recovery dumps cold water faster than the heaters can carry (0.27 L/min of 18C feed ~ 2.6 kW > the 2 kW makeup) -- on the rig this crashed the evap temp under the blower gate and the run limit-cycled. The level-correction terms fold back to production-match as yEvapTemp sags: full correction at set-3, production-match only by set-8. Level can wait; heat can't.

## 2.8  Feed-side inventory dead-reckon (feedInvEst)

Positive-displacement feed pumped IN minus calibrated distillate OUT -- a level estimate that needs no sensor. While venting (meter ~0) it subtracts the blower estimate instead, so it tracks through the steamy startup.

feedInvEst += feedInMl - vaporOutMl     ; vaporOutMl = metered (liquid) | blower est (steam)

## 2.9  Condensate -- single point sensor, pump-to-dry (CondensatePump)

No separate catch tank: condensate collects in the bottom TUBE of the HX. One point-level sensor ~3/4 up the tube (AI:CondLevel > condLevelThresh -> yCondHigh) starts the pump. The flow meter sits on the PUMP OUTLET, so it directly meters distillate leaving. The pump runs until the tube is DRY: when it sucks air the metered flow drops to the incoming trickle (below condEmptyFlow) -- that sustained drop, debounced, is the empty signal. Pumped volume per cycle is the distillate batch; batch / fill-time is the production rate.

start pump when  AI:CondLevel > condLevelThresh   (tube ~3/4 full)

Stop on WHICHEVER fires first (the pump is positive-displacement, so the empty point is bracketed three ways):

(1) yCondFlow < condEmptyFlow for condEmptyDebounce   (meter drops to ~0 on air -> dry; primary)

(2) metered batch > condBatchMaxML                    (guards a meter that keeps counting on air)

(3) pump run > condPumpMaxRun                         (hard time cap -> sets condPumpFault)

condRateLpm = condBatchMl / fill-cycle-time     ->  ProductionControl (2.6)

### One counter, measured in software (condUseTotalizer)

The CTR0 count is read once and exposed as a first-class CTR input -- the cumulative total: CTR:CondTotal = count/K (rollover-safe 32-bit, summed into a big-int). The flow RATE is derived from the total in the expression, windowed over condRateWin (so per-tick pulse coarseness averages out), held between updates:

yCondFlow = (yCondTotal - prevWindowTotal) / windowElapsed * 60     // L/min, every condRateWin

Two ways to report the distillate, both off that one total:

condUseTotalizer = 1 (default):  condProduct = yCondTotal * condCalFactor * 1000   // EXACT, no integration error

condUseTotalizer = 0:  condProduct += integral(yCondFlow) * condCalFactor          // windowed-rate integration

The pump batch cycle and the empty detection (on yCondFlow) are identical either way; the switch only changes how condProduct is formed. condCalFactor (bench: known volume / metered pulses) corrects both. condEmptyDebounce should be >= ~2x condRateWin.

The vent (DO:VentValve) is a startup NON-CONDENSABLE (air) purge: it holds OPEN while the system warms (blower on, phase >= HEAT) so trapped air is pushed out, then CLOSES once the blower-out vapor (yVaporOut) reaches steam temperature -- hysteresis ventOpenTemp (98C, open below) / ventCloseTemp (101C, close above). It is independent of condLiquid (the condensate-outlet liquid gate the sim uses for metering); closed when idle.

## 2.10  Blower soft-start + anti-surge bypass (BlowerControl)

The blower stays OFF below blowerStartTemp (90 C): a cold evaporator has no vapor to pump -- the blower would only pull vacuum. Once the gate opens it starts at blowerRampRpm0 (200 rpm) and ramps at blowerRampRpmS (10 rpm/s) to the target; blowerCmdRpm (the rate-limited state) is the ONLY rpm sent to the VFD, and everything downstream (SH-heater gate, wet-comp arming, production anti-windup) keys off it. This replaced a suction-lie failure on the rig: a hard blower start pulled the level gauge down, the control over-filled chasing the false low reading, and the demister flooded. The HARDWARE fix -- venting the gauge to the evap vapor space (connected-vessel manometer) -- removed the need for pressure compensation entirely; the soft ramp is kept as good practice.

run = runSystem AND !trip AND phase>=HEAT AND yEvapTemp >= blowerStartTemp

cmd = min( max(cmd, blowerRampRpm0) + blowerRampRpmS*dt , blowerRpmSafe )   -> VFD

bypassV = clamp( (surgeThresh - deltaT) * surgeGain , 0 , 10 )   ->  AO:BypassValve

# 3.  Plant Model Equations (MVRSim)

The lumped first-order simulator. Active only when simEnable=1. cp ~ 4186 J/kg.C; 1 L ~ 1 kg of water.

## 3.1  Blower & production

simBlowerRpm += (cmd - simBlowerRpm) * dt/simTauBlower

blowFrac = simBlowerRpm/simBlowerRated ;  compEff = blowFrac*(1-bypass)

boilReady = clamp((eT-95)/5,0,1)

simProd  -> compEff * simVaporMax * boilReady * surge      (L/min, blower-driven)

## 3.2  Pressures -- compressor suction / discharge (Antoine)

The blower is the COMPRESSOR: it draws vapor OFF the evaporator (suction, low side) and discharges into the steam chest (high side). The condensate / discharge side is vented (catch-tank vent), so it rests near atmospheric and rises with the condensing temperature. The EVAPORATOR is the suction side -- it never goes positive, and the instant the blower spins up it is pulled BELOW atmospheric (deepest while boil-off still lags blower throughput), recovering toward atmospheric as the chest warms. Blower off (warm-up): PR=1, both sides rest at atmospheric (air + vent). NOTE: evaporator suction no longer disturbs the level reading -- the gauge vent line ties to the evap vapor space (connected-vessel manometer), so both legs see the same pressure and the sender reads true.

PR        = 1 + (simPRmax-1)*compEff

discharge = max( Psat(simSteamChest) , simAtmPress )    // steam / condensing side

simSteamPress -> discharge        (lifts above atmospheric as the chest heats)

simEvapPress  -> min( discharge / PR , simAtmPress )    // suction: NEVER positive

Tsat(P) via Antoine ;  deltaT = Tsat(steam) - Tsat(evap)   (evap Tsat drops under vacuum)

## 3.3  Superheat

shPower  = shDuty * simSuperPow * simSHmeshEff

shTarget = shPower / (vapor mass flow * simSteamCp)   ;  simVaporIn = eT + simSuperheat

## 3.4  HX condensing & latent recovery

The produced vapor condenses in the HX, releasing latent heat; simRecovEff of it boils the next slug (the recovery). Bidirectional conductance so the slab tracks the evaporator during warm-up.

QinHX  = simProd * simLatentPerLpm * simRecovEff

QoutHX = simUAsteam * (simSteamChest - eT)

simSteamChest += (QinHX - QoutHX)/simChx * dt

## 3.5  Evaporator energy balance

Qmakeup = mkDuty*simMakeupPow ;  Qsteam = QoutHX ;  Qboil = simProd*simLatentPerLpm

Qfeed   = feedFlow*(cp/60)*(eT - feedEntry) ;  Qloss = simUAloss*(eT - simAmbient)

simEvapTemp += (Qmakeup + Qsteam - Qboil - Qfeed - Qloss)/simCevap * dt

## 3.6  Subcooling HX  (effectiveness-NTU)

After condensing, the liquid is subcooled by the cold feed in a counterflow section. The outlet temp falls out of the HX area (simUAsub) and the flows -- not a fudge. A slab thermal mass (simChxSub) makes the output STEAM at startup (hot slab) until the feed cools it.

feedC = feedFlow/60 * cp                  (feed capacity rate, W/C)

NTU   = simUAsub / feedC ;  eff = NTU/(1+NTU)

condOutSS = simSteamChest - eff*(simSteamChest - feed)

RUN:  simHXbottom += (condOutSS - simHXbottom) * (feedC/simChxSub)*dt

simCondTemp = simHXbottom

## 3.7  Feed preheat recovery  (energy-consistent)

The sensible heat the condensate loses in the subcool HX is exactly the heat the cold feed gains (other side of the same HX). The preheated feed enters the evaporator, reducing the makeup load -- one consistent energy flow with 3.6.

feedPre  = (simProd/feedFlow) * (simSteamChest - simHXbottom)

feedEntry = clamp( feed + feedPre , feed , eT )

## 3.8  Condensate tank, flow meter, steam vent

The catch tank fills with LIQUID only (condLiquid); steam vents (and spins the meter, which the control ignores). HX holdup adds a formation delay; the pump cycles between the two level switches.

## 3.9  Cartridge surface temps (heater TCs)

makeup surface = eT + mkDuty*( covRise + uncovRise*(1 - coverFrac) )   (spikes when uncovered)

super surface  = simVaporIn + shDuty*simSuperRise

# 4.  Calibration Tests

How to pin each model constant from the real assembly. Run these once the rig is built; they let the sim match measured behaviour. Use the app's logging (the CSV captures every gvar_* static) and read the steady values.

### 4.1  Heater coast-down  ->  simUAloss  (and a thermal-mass check)

Determines: Standby heat-loss coefficient.

Procedure:

- Bring the evaporator to a steady temperature (e.g. ~95 C).

- Turn OFF all heaters, blower and feed. Log the evaporator temp as it cools toward ambient.

- Record the cooling curve T(t) and the ambient temp.

Compute:

tau = time constant of the exponential cool-down (T-ambient) vs t

C   = simCevap + simChx  (total mass);   simUAloss = C / tau

or from the initial slope:  simUAloss = C * (dT/dt) / (T - ambient)

### 4.2  Warm-up rate  ->  simCevap (+ simChx)

Determines: Evaporator + HX thermal mass.

Procedure:

- From cold, full makeup power (known simMakeupPow), no blower / no production.

- Log the evaporator temp vs time over the linear part of the warm-up.

Compute:

dT/dt (degC/s) measured over the rise

C = (simMakeupPow - Qloss) / (dT/dt)        (Qloss small early)

Split: simChx ~ steel mass*0.5 + water mass*4.2 (kJ/C); simCevap = the rest

### 4.3  Production vs blower RPM  ->  simVaporMax, prodPerRpm, simPRmax, simCompGain

Determines: Blower-to-production and the pressure-ratio / deltaT relationship.

Procedure:

- At steady boil, set several blower RPMs (e.g. 1000, 1500, 2200, 3000).

- At each, log steady distillate rate (calibrated condProduct slope), deltaT (steamTsat-evapTsat), and vapor-out temp.

Compute:

simVaporMax  = production at full blower / full boil (extrapolate)

prodPerRpm   = slope of production vs rpm (control-side estimate)

simPRmax     from deltaT at full blower ;  simCompGain from (VaporOut - condensing temp) vs (PR-1)

### 4.4  Steady subcool  ->  simUAsub

Determines: Subcool-HX conductance (area).

Procedure:

- At steady production, log the condensing temp (steam chest TC), the feed inlet temp, the condensate outlet temp (TC:CondTemp), and the feed flow.

Compute:

eff = (condensing - condOut) / (condensing - feed)

NTU = eff/(1-eff)          (balanced counterflow)

simUAsub = NTU * (feed mass flow * cp)        feedC = (L/min)/60 * 4186

### 4.5  Steam-phase length  ->  simChxSub

Determines: Subcool-zone slab thermal mass.

Procedure:

- From a cold start at fixed production, time how long the condensate OUTLET stays above boiling (steam) before it drops below ~boiling and liquid starts collecting.

Compute:

t_steam ~ (simChxSub/feedC) * ln( (Tstart-condOutSS)/(98-condOutSS) )

->  simChxSub = feedC * t_steam / ln(...)   (or just tune simChxSub to match the observed time)

### 4.6  Recovery  ->  simUAsteam

Determines: HX latent-recovery conductance.

Procedure:

- At steady RUN, log the steady makeup power, production, feed temp, evaporator temp, and steam-chest temp.

Compute:

Energy balance:  Qmakeup + Qsteam = Qboil + Qfeed + Qloss

Qsteam = simUAsteam*(steamChest - eT)  ->  solve for simUAsteam from the measured makeup

### 4.7  Condensate flow meter + tube  ->  condCalFactor, condEmptyFlow, condTubeTripML

Determines: Pump-outlet flow-meter calibration, the dry threshold, and the tube/sensor volume.

Procedure:

- Bench: pump a known liquid volume through the meter; record pulses (or the app's integrated raw) to get pulses/L -> condCalFactor.

- Run the condensate pump empty (dry) and note its steady flow reading, then pumping liquid and note that reading -- the dry threshold sits between them.

- Fill the HX bottom tube to the sensor trip and measure the volume pumped out (the app shows condBatchMl).

Compute:

condCalFactor = known_volume / metered_raw     (bench, fixed)

condEmptyFlow ~ halfway between the dry trickle (~production) and the full pump rate

condTubeTripML = measured batch per cycle (geometric ~130 ml at 3/4 of the 1.16in x 10in tube)

### 4.8  Pump cal  ->  feedLPerStep   (already done)

Determines: Peristaltic feed-pump volume per motor step.

Procedure:

- Run the pump a known number of revolutions into a graduated container; measure the volume.

Compute:

feedLPerStep = volume(L) / (revs * feedStepsPerRev)     e.g. 0.200 / (223*10000) = 8.969e-8

### 4.9  Cartridge trip points  ->  makeupHtrMax, superHtrMax

Determines: Safe over-temp trip thresholds for the heater TCs.

Procedure:

- Run normally and log the makeup- and superheat-cartridge TCs (ch6/ch7) across the operating range.

- Note the highest NORMAL surface temp each reaches.

Compute:

set makeupHtrMax / superHtrMax a comfortable margin (e.g. +20-30 C) above the highest normal reading;

makeup must trip before the element can run dry (uncovered) -- verify by lowering the level.

# 5.  Variable Reference

Every static, grouped by role. Temps degC, pressures psia, flows L/min, volumes ml unless noted.

## 5.1  Operating setpoints (the knobs you set)

| Variable | Default | Range | Meaning |
| --- | --- | --- | --- |
| runSystem | 0 | 0 / 1 | Master run (RUN button). 0 = everything idle. |
| productionSet | 0.12 | 0.02 - 0.24 L/min | PRIMARY KNOB: target distillate rate. Blower + feed chase it. Out-of-range ignored. |
| evapTempSet | 100 | 95 - 105 C | Evaporator boiling setpoint (makeup-PI target). |
| superheatSet | 3 | 1 - 8 C | Vapor superheat setpoint (superheat-PI target). |
| blowerRpmSet | 1500 | 800 - 4000 | Blower rpm. Auto when autoBlow=1, else manual. |
| feedRpmSet | 60 | 0 - 300 | Feed pump rpm. Auto when autoFeed=1, else manual. |
| evapLevelSet | 2475 | 500 - 3500 ml | Operating level the feed holds (= evapMidMark, the 2350¦2600 plateau edge, ~2.5 L). Lowered from 3025: thermal expansion (~4% cold->boiling) + wobble took the 3/4 fill over the top on the rig. |
| autoBlow / autoFeed | 1 / 1 | 0 / 1 | Enable the blower / feed closed loops. |
| emptySystem / shutdown | buttons | 0 / 1 | Button widgets (Variable mode): reverse-pump the system dry WITHOUT a run -- to sender bottom + purgeExtraML (emptyMaxML backstop). Heaters are already forced off with Run off, so shutdown = verified-heaters-off + pump-out. IGNORED while running (turn Run off first); toggle off to re-arm. |

## 5.2  Safety bounds, interlocks & trips

| Variable | Default | Units | Meaning |
| --- | --- | --- | --- |
| prodSetMax | 0.3 | L/min | productionSet rejected above this (or below 0); last valid kept (prodSetSafe). |
| superheatSetMax | 10 | C | superheatSet upper bound (-> superheatSetSafe). |
| evapTempSetMin/Max | 90 / 110 | C | evapTempSet window (over-pressure guard) (-> evapTempSetSafe). |
| blowerMaxRpm | 3500 | rpm | Blower clamp (-> blowerRpmSafe). Lowered from 4000: in the stuck-hot deadlock the integral wound to 4000 and just churned the same steam hotter. |
| blowerRunMinRpm | 1950 | rpm | Production-control TURNDOWN FLOOR: once running, the blower target never drops below this -- below ~2000 there is no real vapor flow, superheat dies, and the run's thermal progress unwinds. If production exceeds productionSet at the floor, the feed simply matches it. (blowerMinRpm 800 remains the absolute floor used by gates elsewhere.) |
| feedRpmMax | 300 | rpm | Feed-pump clamp (-> feedRpmSafe). |
| makeupHtrMax | 140 | C | Makeup-cartridge TC over-temp HARD trip (backstop). SOFT fold-back first: above makeupHtrSoft (115) the duty scales down over makeupHtrFoldBand (15), never below makeupHtrMinDuty (0.12). Latched. |
| superHtrMax | 275 | C | Superheat-cartridge TC over-temp TRIP -- hard backstop above the 250C element regulation (the cartridge runs HOT by design in thin vapor). Latched. |
| superHtrRegC / kpShElem | 250 / 0.05 | C / duty/C | SH ELEMENT-TEMP REGULATION: duty capped so the cartridge holds ~250C until real vapor flow carries the heat away (poor coupling into thin vapor); then the superheat PI takes over. Raised 200 -> 250 per rig (200 couldn't keep up). Full duty 20C below target. |
| vaporOutSoft | 120 | C | Blower VAPOR-OUT FOLD-BACK: above this discharge temp, ALL blower raises pause (integral/seek/lift) and the target actively backs off -- the blower is heating the vapor, not pumping it (rig 7/15: vO rode 110-138 against the 140 trip while nothing condensed and every other gate read green). |
| prodStallLpm / prodStallS / ventPulseS | 0.03 / 300 / 30 | L/min / s / s | AIR-ACCUMULATION PURGE: RUN production stalled below prodStallLpm at temp for prodStallS -> the vent PULSES ventPulseS to dump the non-condensable blanket, repeating while stalled. Feed water carries dissolved air in continuously; air collects exactly on the condensing surface, and the steam side then runs 1-3 psi above Psat(T) -- the pressure-derived deltaT looks healthy while the true thermal dT is ~0 and nothing condenses (rig 7/15). |
| ventReliefC | 125 | C | THERMAL-RELIEF vent: above this vapor-out the vent OPENS regardless of the ventDone air-purge latch -- dumps the superheated charge so fresh saturated vapor pulls through (superheated gas will not condense; the manual remedy that recovered the rig). Hysteresis: closes below this -10. |
| maxEvapPress / maxSteamPress | 25 / 35 | psia | Over-pressure trips. |
| maxVaporOut | 140 | C | Vapor-out over-temp trip. |
| minSuperheat / superHFaultS | 0.5 / 20 | C / s | Wet-compression (low-superheat) trip. Fires/arms only at REAL speed (target*0.9 AND >= blowerMinRpm) AND with the evaporator at temp (>= set-5); DISARMS whenever the blower leaves at-speed (restart ramps legitimately collapse superheat); superheat must sit below the floor for superHFaultS CONTINUOUSLY (swings dip briefly, wet compression persists); and the sag does NOT count while the ELEMENT REGULATOR caps the duty (shElemLim: a 200C mesh can't be wet -- that sag is a capability limit; real wet comp cools the mesh, the cap releases, and the sag then counts). Latched. |
| deltaTMin | 2.5 | C | MINIMUM COMPRESSION LIFT: deltaT (steamTsat - evapTsat) is the MVR's working head -- at ~1C the HX moves no heat, nothing condenses, and the sealed system just pressurizes. Below this (at temp, vent closed, HEAT or RUN) the blower target RAISES toward the seek cap; the superheat rescue only acts when lift is adequate. |
| feedStallRpm / feedStallMaxML | 200 / 1500 | rpm / ml | FEED-STALL guard: this much feed dead-reckoned at fill-rate commands without one UPWARD sender step -> the pump is not delivering (empty supply/tubing/stepper) or the sender is stuck -> latches evapLevelErr (trip + popup). Rig 7/14: 300rpm for 15min, ~4L commanded, level frozen, vessel slowly pressurizing. |
| tsatTrimC | 1.4 (calibrate) | C | TSAT ALIGNMENT TRIM added to both computed Tsats: TCs + pressure transducers carry small offsets (rig: the evap TC read ~1.4C above Tsat(eP) at a rolling boil -- computed superheat inflated, boil needed eTset 101.5+). CALIBRATE at a vigorous VENTED boil: tsatTrimC = yEvapTemp - evapTsat. Cancels in deltaT; lowers superheatIn by the trim. |
| feedMinLpm | 0.03 | L/min | MINIMUM RUN FEED: the cold feed IS the subcooler's heat sink -- at feed 0 nothing condenses, production reads 0, the level never drops, and level control correctly asks for zero feed (a self-sustaining deadlock the level loop cannot see; rig 7/15). A trickle keeps the condensing path alive; the high-level cut overrides. The feedLevelTrim negative bound is -0.05 for the same reason (it only ever needs to shave feed-forward bias). |
| feedSlewRpmS | 10 | rpm/s | AUTO-FEED SLEW LIMIT: production-measure jumps slammed the feed 0..450rpm, and every slam is a cold-water rush the makeup PI then chases (the 42s eT limit cycle). The level loop is slow anyway -- the command is rate-limited. |
| evapHighMark | 2750 | ml | HIGH-LEVEL WARNING (2600¦2900 plateau edge, trips on the 2900 plateau; E-TC DIN float ORed in): latched UI popup + FeedFollow cuts the feed. NOT during PURGE -- an overfull warm start reads HIGH while the purge is draining it. Not a hard trip. |
| senderFaultS / senderErr | 2 / latch | s | Level-sender open/short (V outside -0.3..3.3) while running, debounced -> LATCHED trip + popup (a dead sender mid-run would read 'all dry' and slow-flood). Clears on stop. |
| purgeStallS / purgeErr | 240 / latch | s | PURGE watchdog: level must keep dropping while purging with a good sender (disarmed once bottom is reached). Stall = pump/direction fault -> LATCHED trip + popup. |
| Flood trip | AND gate | - | DemisterFlood counts only when DemisterWater is ALSO wet: flood physically sits above the demist sensor, so a real flood always has both; fog/condensation films the flood sensor alone (false-tripped on the rig). |

## 5.3  Control tuning

| Variable | Default | Units | Meaning |
| --- | --- | --- | --- |
| kpMakeup / kiMakeup | 0.5 / 0.03 | - | Makeup-heater PI gains. 0.05 was integral-only (rail-to-rail duty, eT +/-2.5C); the user's 0.7/0.07 gave a fast 42s +/-1.2C limit cycle against the ~10s TC lag. 0.5/0.03 + the FEED SLEW LIMIT (the real disturbance was cold-water feed rushes) is the balance. |
| kpSuper / kiSuper / kShFF | 0.02 / 0.002 / 0.03 | - / - / duty/krpm | Superheat control = FLOW FEEDFORWARD + PI trim. The duty operating point scales with vapor mass flow (0.05 holds ~5C at 1500rpm, the same duty is nothing at 4000 -- the rig sagged into the wet-comp trip while the PI crawled), so kShFF*rpm/1000 carries it and the PI (retuned soft after the 0.05/0.01 gains bounced the element 173..209C) only trims; the PI error is CLAMPED to +/-10C (a cold pipe reads -74 -- not a deficit the cartridge can fix; unclamped it slammed duty to 1.0 and overshot the element to 269C) and the integral clamps +/-0.4 so it can cancel excess FF. The blower also actively RESCUES superheat: collapsed superheat at temp (vent closed) backs the rpm target off toward the run floor -- less mass flow = more superheat per watt. |
| kProd | 20 | rpm/(L/min.s) | Blower production-control gain. Anti-windup: the target is not raised while the soft-start/ramped command still lags it. |
| kFeedLevel / kFeedLevelI | 0.015 / 0.0015 | (L/min)/ml, (L/min)/s | Feed level P gain (active OUTSIDE the deadband: fill + upset recovery) / slow edge-ride integral (INSIDE the deadband; the tight hold). |
| levelDeadML | 100 | ml | P deadband around the setpoint: P silent inside (the EvapMid edge integral rules), active outside. |
| senderSpanML / kLevelBias | 200 / 0.0003 | ml / (L/min)/ml | Fusion window half-width (estimate clamped to plateau center +/- this; no teaching) / rate-bias learning gain (only plateau-CHANGE midpoint snaps teach). |
| blowdownFrac | 0 (disabled) | - | Concentrate blowdown DISABLED -- no blowdown valve fitted, so feed exactly matches distillate. Dissolved solids concentrate over a run; drain/flush the evaporator periodically (manual blowdown). Set >0 only with a real bleed/drain. |
| prodPerRpm | 0.00007 | (L/min)/rpm | Blower->production estimate (feedInvEst while venting). Calibrate. |
| surgeThresh / surgeGain | 1.0 / 5.0 | C / (V/C) | Anti-surge bypass: opens below this dT, this many V/C. |
| condLevelThresh | 2.5 | V/units | AI:CondLevel above this = tube ~3/4 full -> start the condensate pump (the point sensor trip). |
| condUseTotalizer | 1 | 0/1 | Distillate measurement: 1 = exact cumulative pulse totalizer (CTR:CondTotal, count/K); 0 = integrate the windowed rate. |
| condEmptyFlow | 0.25 | L/min | Pump-outlet flow below this = tube DRY. Set between the ~0 dry reading and the condensate-pump rate (0.5). |
| condEmptyDebounce | 2.5 | s | Dry must persist this long before the pump stops. Keep >= 2x counter_window_s (the meter's rate window). |
| condBatchMaxML | 450 | ml | Stop the pump if a single metered batch exceeds this (backstop for a meter that keeps counting on air). |
| prodUseInput / prodInWinS | 0 / 240 | 0-1 / s | INPUT-SIDE production measurement OPTION: prod = calibrated feed IN minus the sender-anchored level CHANGE over the window -- only the positive-displacement pump + fused level, nothing from the condensate meter (for when the output measurement can't be trusted; in sim the input estimate beat a 12%-miscalibrated meter). prodUseInput=1 selects it for the blower production control; 0 = output measurement (default). All downstream logic (integral, floor, seek, caps) shared. |
| prodWinS / prodFromTotal | 180 / 1 | s / 0-1 | TOTALIZER-WINDOW production rate: with prodFromTotal=1 (default) condRateLpm comes SOLELY from the cumulative meter delta over this rolling window -- needs NO pump cycles (a stuck-wet CondLevel micro-cycles the pump so no batch ever reaches condRateMinML; the rig sat blind at the seek cap with a WORKING meter). Batches still refresh the same rate more precisely when they land; >= 5ml/window required, plausibility-capped. |
| condTimedEn / condTimedOnS / condTimedOffS / condTimedMinLpm / condTimedAlways | 1 / 5 / 30 / 0.05 / 1 | - | TIMED FLOW-FOLLOW pumping (rig-proven): probe every condTimedOffS (30s), run at least condTimedOnS (5s), then KEEP pumping while the 5s-avg meter flow holds above condTimedMinLpm -- the METER decides when the reservoir is empty, not the fogged CondLevel sensor. condTimedAlways=1 (DEFAULT) = primary strategy from RUN start; 0 = sensor-driven with this as the automatic sensor-bad fallback (condSensErr popup: pump cycled condMeterFaultS with no real batch but the meter counted flow). Timed cycles are CLEAN pump cycles, so batches + the batch rate still work. |
| condMeterFaultS | 240 | s | MEASUREMENT-DEAD warning: cumulative pump-ON time since the last real batch above this with ~no metered volume -> condMeterErr popup (pump airlocked, meter dead, or CondLevel stuck wet micro-cycling the pump). Production control is BLIND while this persists (blower holds at the seek cap). Warning, not a trip; clears on a real batch or Stop. |
| condRateMinML / condRateMaxLpm | 50 / 0.25 | ml / L/min | Production-rate SANITY (rig 7/13): batches under condRateMinML are noise (pump-prime lag trips the dry-detect at ~1ml) -- no rate is computed and the fill timer KEEPS RUNNING (micro-batches resetting it made a real 458ml batch read an impossible 0.75 L/min and the blower turned down 3400->800 chasing it). Computed rates above condRateMaxLpm (~28kW-equivalent) are discarded; the last good rate is kept. |
| condPumpMinRun / condPumpMaxRun | 1 / 120 | s | Min run before dry-detect is armed / hard time cap (sets condPumpFault) if it never goes dry. |
| condLiquidTemp | 98 | C | Condensate outlet below this = LIQUID (count meter, sim collects condensate). Not the vent anymore. |
| ventOpenTemp / ventCloseTemp | 97 / 100 | C | Startup air-purge vent: OPEN below / CLOSE above this blower-out vapor temp (yVaporOut). Lowered from 98/101 -- the rig never reached 101 at the vent TC, so it sat open bleeding steam. Once closed it LATCHES closed for the rest of the run (ventDone): the air purge is once-per-run; a mid-run upset cooling yVaporOut must not re-open it. ProductionControl does not integrate while the vent is open (production is being vented -- the meter can't see it). |
| ventBoostRpm | 2500 | rpm | VENT-PURGE BOOST: blower target while the vent is open with the evaporator at temp (>= set-3). At the nominal 1500 the vapor mass flow is too thin to heat the discharge path -- VaporOut crawled 65->84C in 14min on the rig and the vent never closed. Boost shoves mass through; when the vent closes the target is SEEDED at the boost rpm (hand-off, no step down -- dropping 2500->1500 crashed the progress on the rig) and glides to the natural operating point under production/pressure control. |
| evapPressMin / blowerSeekRpmS / blowerSeekMaxRpm | 13.0 / 25 / 2800 | psia / rpm/s / rpm | Blower PRESSURE-SEEK (RUN, vent closed, BEFORE the first condensate batch): no measured production exists yet, so instead of freezing at the nominal 1500 the target CLIMBS at blowerSeekRpmS while the suction holds above evapPressMin+0.3 AND superheat is healthy (> minSuperheat+1) -- superheat collapse is the other 'outpacing the plant' signal -- CAPPED at blowerSeekMaxRpm (the rig boils hard enough that suction never sagged and the seek ran to the 4000 clamp). After batches, the production integral takes over (may exceed the cap) but never raises the target with suction at the floor; either mode backs off below the floor. |
| DemisterWater / DemisterFlood / CondLevel | wet-dry | 0-5V | ALL WET/DRY point sensors (0=dry, 5V=wet), like the Evap sensors. Muxed to 0/1 (real: AI > 2.5V). Mist = water at the demister; Flood = overflow above it; CondLevel = condensate at the tube sensor (~3/4 up). |
| demisterDrainRunS / demisterPumpMlMin | 12 / 500 | s / ml/min | Demister drain peristaltic pump: ON when DemisterWater wet, runs at least demisterDrainRunS to clear below the sensor (single wet/dry threshold + min-run = no chatter), then off when dry. (sim trip/flood levels demisterTripML 80 / demisterFloodML 150 cc set where the sim sensors go wet.) |
| sysMs | (server) | ms | System monotonic clock, stamped by the server each tick BEFORE evaluate. Read it for any interval math (now - startMs). |
| dtReal | ~timeIncSec | s | REAL elapsed time since the last tick = (sysMs - lastTickMs)/1000, guarded. ALL expression timing (PI integrals, condensate timers, sim dt) uses this, not timeIncSec. |
| timeIncSec | 0.00625 | s | Nominal tick period -- now only the FALLBACK for dtReal on the first ticks / a scheduling hiccup. |

## 5.4  Startup, pump cal & feed-side estimate

| Variable | Default | Range/Units | Meaning |
| --- | --- | --- | --- |
| mvrPhase | 0 | 0=IDLE 1=PURGE 2=PRIME 3=HEAT 4=RUN | Startup state machine. |
| feedLPerStep | 8.96861e-8 | L/step | FEED PUMP CAL: L per motor step (200mL/223rev/10000steps). |
| feedStepsPerRev | 10000 | steps/rev | Match driver microstep Pr0.00. L/min = rpm x steps x LPerStep. |
| feedPrimeML | 750 | ml | PRIME fills to this (= EvapLow mark, heater covered) + confirms wet, THEN heats. Feed ramps to the operating level during warm-up. |
| feedPrimeRpm | 250 | rpm | Feed rpm during the PRIME fill (~224 mL/min). |
| purgeRevRpm | 400 | rpm | PURGE / EMPTY / SHUTDOWN: feed-pump REVERSE rpm (~359 mL/min). |
| purgeWarmEn | 1 | 0 / 1 | WARM-START purge: sender shows water at run start -> pump down until the level has dropped AT LEAST ONE plateau (the drop-edge crossing snaps/calibrates the estimate) AND reads at/below the operating mark (an overfull-shutdown restart keeps pumping down to the mark -- coming from above, the last crossing IS the 2475 edge). Then PRIME/HEAT continue from that calibrated level -- no purge-to-dry + refill. 0 = always full purge. |
| purgeExtraML | 200 | ml | FULL purge only: after the sender reads BOTTOM (plateau 0), keep reverse-pumping this much more (dead-reckoned) -- water below the float pickup is still in there. |
| purgeMaxML | 2000 | ml | FULL-purge backstop: blind reverse-pump cap, used only if the level sender is faulted (normally the purge stops at bottom + purgeExtraML, or at the mark on a warm start). |
| emptyMaxML | 4000 | ml | EMPTY/SHUTDOWN drain backstop: must exceed a FULL operating charge (~2.5 L) + purgeExtraML. (purgeMaxML is sized for the startup residual and would cut a full-charge drain short.) |
| yFeedEmpty / purgeEmptyML | sensor / 50 | - / ml | Input level detector = empty (real sensor; sim: evaporator inventory below purgeEmptyML). Monitoring only. |
| evapLowMark | 750 | ml | LOW mark (587¦850 plateau edge, physical ~720): heater-covered floor + PRIME wet-confirm. Heater covered ~550ml (simHtrCoverML). |
| evapMidMark | 2475 | ml | OPERATING mark (2350¦2600 plateau edge, ~2.5 L) = evapLevelSet. The feed holds the level here (deadband P + edge integral, 2.2a). |
| Tank-gauge sender (AI6 EvapLevel) | cal table | V -> ml | 33-240ohm arm-float sender + 550ohm (4x2.2k parallel) pullup to REGULATED 12V. AI6 slope -1.0 / offset 3.68 (KEEP: the cal is in these scaled volts; dry=0V, full ~3.0V, uh-oh-full 3600ml). Level-vs-ohms is strongly NONLINEAR (arm swing + tank geometry), so the decode is the MEASURED 14-plateau table (MVR Level Cal.csv) mapping scaled V -> plateau-center ml. Valid range -0.3..3.3V; outside (open wire = negative, short = 3.68) -> all switches DRY, no heat (+ senderErr trip mid-run). Level switches derive from the decoded plateau: low 750, mid/operating 2475, high-warn 2750. feedInvEst FUSED: dead-reckon + plateau-change midpoint snaps (teach the rate-bias) + span window (+/-senderSpanML, no teaching). HARDWARE: the gauge vent line ties to the EVAP VAPOR SPACE (connected-vessel manometer), so the sender reads TRUE level at any evaporator pressure -- keep that vent line sloped so condensate drains (a water slug in it re-creates an offset). |
| condCalFactor | 1.0 (MANUAL) | x | Flow-meter cal -- set ONCE against the feed pump at steady level: distillate = feed/(1+blowdownFrac). Auto-cal removed (it looped through the blower production control). |
| evapLevelCheckML | 1000 | ml | PRIME verify: if this much fill is dead-reckoned and EvapLow is still DRY -> evapLevelErr fault + popup (feed-pump or sensor failure). |
| feedInvEst | 0 (->~2475) | ml | Feed-side inventory estimate: dead-reckon fused with the sender (snaps + window). Seeded from the sender at run start and purge exit -- never fiat-zeroed. |
| blowerStartTemp / blowerStopTemp / blowerRampRpm0 / blowerRampRpmS / blowerRampHotS | 90 / 80 / 200 / 10 / 50 | C / C / rpm / rpm/s / rpm/s | Blower SOFT-START with HYSTERESIS: starts at blowerStartTemp (cold evaporator has no vapor -- it only pulls vacuum), ramps from blowerRampRpm0. TWO-STAGE ramp: blowerRampRpmS while warming (thin vapor), blowerRampHotS once the evaporator is at temp (>= set-3; 200->1500 in ~26s). Once RUNNING it stops only below blowerStopTemp -- the pipe TC hovers near 90 with flow, and a single 90C gate limit-cycled the blower on the rig. blowerCmdRpm is the ONLY rpm sent to the VFD. |
| condTubeTripML | 300 | ml | Condensate reservoir level at the wet/dry sensor (~3/4 up) = pump-START. The nominal distillate batch. |
| condTubeFullML | 400 | ml | Full condensate reservoir volume. |

## 5.5  Simulator plant parameters

| Variable | Default | Meaning |
| --- | --- | --- |
| simEnable / simDriveHW | 0 / 0 | Sim master / hardware-in-the-loop. |
| simSpeed | 120 | Sim time-acceleration x real (dt clamps at 1.0s). |
| simAmbient / simFeedSupply | 25 / 18 C | Ambient / cold feed-water supply temp. |
| simMakeupPow / simSuperPow | 2000 / 300 W | Makeup heater / superheat cartridge ratings. |
| simSHmeshEff / simSteamCp | 0.4 / 2000 | SH mesh-to-steam fraction / steam cp (J/kg.C). |
| simUAsteam | 1500 W/C | HX condensing -> evaporator LATENT recovery conductance. |
| simChx | 14000 J/C | Main HX thermal mass (24 lb steel + ~2L water) -- warm-up lag. |
| simCevap | 60000 J/C | Evaporator thermal mass -- warm-up lag. |
| simUAloss | 3 W/C | Standby heat-loss coefficient (insulated rig). Drives simHeatLoss. |
| simPRmax / simCompGain | 1.25 / 60 | Blower pressure ratio / compression superheat (C per PR-1). |
| simAtmPress | 14.7 | Atmospheric / vent reference (psia). Evaporator pressure is floored here (non-condensable air + vent) until Psat(eT) exceeds it at boiling, so it stays flat through warm-up then lifts at Tsat. Set to local barometric. |
| simBlowerRated | 3450 rpm | Blower rated speed (blowFrac = rpm/this). |
| simVaporMax | 0.25 L/min | Vapor production at full blower + full boil (production scale). |
| simLatentPerLpm | 38000 W | Latent heat per L/min of production (~ real water). |
| simRecovEff | 0.97 | Latent fraction recovered (HX condensing + compressor work). |
| simUAsub | 50 W/C | SUBCOOL-HX conductance (U.A) -- sets how cold the condensate gets. |
| simChxSub | 8000 J/C | Subcool-zone slab thermal mass -- sets the steam-phase length. |
| simTauBlower / simTauP / simTauSH | 2 / 3 / 5 s | Blower spin-up / evap pressure / superheat time constants. |
| simHtrCoverML | 550 ml | Evaporator inventory above which the makeup cartridge stays covered (inlet tube + fittings - heater displacement, element top ~9.5 in up). |
| simMakeupRise / simSuperRise | 150 / 300 C | Cartridge surface rise per duty (uncovered makeup / superheat). |
| simLevelBobML | 30 ml | Float bob from the boil (RMS) on the simulated sender reading. |
| simSpillML | 3800 ml | Sender-referenced level where overfill SPILLS into the demister (past totally-full 3.6 L + fittings). |
| condCoolDuty | 0.4 | (legacy) makeup-duty cool threshold -- superseded by the subcool-HX model. |
| simPumpMlMin | 500 | Condensate-pump flow rate (ml/min, = real 0.5 L/min) -- must exceed production so the pump empties the tube; also the sim meter reading while pumping liquid. |
| simFlowCalErr / simFlowNoise | 1.12 / 0.05 | Flow-meter mis-cal (reads ~12% high) / rate noise. |
| simSurgeAmp / simFeedNoise | 0.06 / 2.0 | Production surge amplitude / feed-temp fluctuation (C RMS). |

## 5.6  Simulator state (model outputs -- chart these)

| Variable | Meaning |
| --- | --- |
| simEvapTemp | Evaporator temperature. |
| simSteamChest | HX condensing (TOP) temp -- the 'steam chest'. |
| simHXbottom | HX cold feed-inlet (BOTTOM) = condensate outlet temp. |
| simCondTemp | Condensate outlet temp (= simHXbottom; the TC:CondTemp reading). |
| simVaporIn / simVaporOut | Vapor temp in / out of the blower. |
| simEvapPress / simSteamPress | Evaporator / steam-side pressure (psia). |
| simProd | Vapor production (boil-off) rate (L/min). |
| simEvapInv | True evaporator inventory (ml); prime fills, feed holds evapLevelSet. |
| simCondVol | Condensate volume in the HX bottom tube (ml) -- sim state. |
| simCondFlow | Sim pump-outlet flow meter (L/min): pump rate while liquid, drops to the trickle when dry. |
| simBlowerRpm | Actual blower rpm (lags the command). |
| condLiquid / ventOpen | Condensate-outlet liquid-vs-steam gate (sim metering) / startup air-purge vent state (DO:VentValve). |
| simHeatLoss / simHeatIn | Standby heat loss / total heat in (W) -- monitoring. |
| condPumpOn / condProduct | Condensate pump command / lifetime calibrated distillate total (ml). |
| condBatchMl / condRateLpm | Last pumped batch (ml) / measured production rate = batch / fill time (L/min). |
| condCalFactor / condPumpFault | Flow-meter calibration (bench, x raw) / 1 = pump hit timeout without going dry. |